VOYAGE ESTIMATION

2021

VOYAGE ESTIMATION TO FIND THE T.C.E.   by N. Pentheroudakis

Our vessel is open 12-14 March ‘19 in Lagos, Nigeria (West Africa) and we are proposed the following order, for which we need the Time Charter Equivalent:

M/V “ DELTA ” 15.838 TDW at 9,10 m. draft, Built 2015, Malta flag, Single/double skin 4/4 HO/HA Gearless 18100 CBM capacity – 12.9 knots (nautical miles per hour) on daily consumption of 15 tons F.O. + 1,5 Ton M.G.O.,  Port consumption daily 1 Ton M.G.O.

Here is the order:

- 15.000 mtons minimum/maximum iron ore

- Conakry, Guinea (West Africa) / Genova, Italy - 1 safe berth both ports

- No draft/length restrictions

- 15-21 March 2019

- 2500 ts loading rate SHINC

- 3000 ts disch SATSHEX - Friday 17:00 / Monday 08:00 not to count EIU

- N.O.R. to be tendered during office hours only 08:00 to 17:00 both ports

- At loading, Laytime to commence 08:00 next working day if NOR tendered at any time during office hours. At discharging, Laytime to commence 14:00 if NOR tendered morning, and 08:00 next working day if NOR tendered afternoon

- Freight USD 26.- /mtn FIOT

- Commision 3.75% total

Required the TIME CHARTER EQUIVALENT, considering also:

- Distances: Lagos-Conakry 1.205 miles and Conakry-Genova 3.120 miles

- D/A load port USD 18,500.- /disch port USD 24,500.-

- Average Bunker prices: Fuel Oil USD 485/tn  -  M.G.O. :USD 690/tn

- On commencement of the voyage (Lagos) bunkers remaining on board are 198 tons F.O. and 31 tons M.G.O.

TIME

LOADING PORT …15000/2500                                            6   DAYS

DISCHARGING PORT 15000/3000                                       5      “

MARGIN AT PORTS (.*1)  1.5+5,5…                                     7      “

EXTRA TIME IN PORT,IF ANY …O,5……                     … .0,5    “

TTL DAYS IN PORT                                                             18.5

DAYS AT SEA    4325 / 12.9 / 24                                       14……  ”

MARGIN AT SEA  ( *2)                                                          1

TTL SEA DAYS                                                                    15

VOYAGE DURATION      …………………………                33.5  DAYS

EXPENSES

COST OF FUEL OIL (*3)…15 X 15 X  $ 485      ….          $ 109,125.-

COST OF MGO AT SEA      15 X 1,5 X $ 690                  $    15,525.-

COST OF MGO IN PORTS    18,5 X 1 X $690                $      12,765.-

CANAL TRANSITING TOLLS,IF ANY (*4)                       $ …………...

PORT CALL EXPENSES                                                 $ 43,000.-

EXTRA COST,IF ANY (*5)                                               $ 4,000.-

TOTAL VOYAGE EXPENSES                                         $ 184,415.-

INCOME

GROSS FREIGHT 15,000 x $26                                    $ 390,000.-

COMMISSION  3.75% .                                                   $ 14,625

NET  FREIGHT……                                                        $ 375,375.-

375,375 - 184,415….…………………………………………

NET INCOME……..                                                 $ 190,960.-

TIME CHARTER EQUIVALENT

190.960 /33,5

….…………….……….   =    $…5,700.-………

===========